- Digital model outputs

The gravity modelling results are provided in the form of digital grids and contour shapefiles.  All these are referred to UTM Zone 31 (ED50).

Model grids

The model grids all have a 2.5 km grid node spacing, but the structural grids were derived by interpolation from input grids with a 5 km node spacing. The grids are provided in ESRI (ArcGIS), ESRI binary (flt), Zmap, Geosoft, and ASCII (xyz) formats. Layer files are included with the first of these which contain the colour symbology used in displaying these grids in the current report.

| ginv_v1_gobs | Observed gravity field (free-air anomalies offshore, Bouguer anomalies onshore) (mGal). Shown in Figure 22 of the report. |
| --- | --- |
| ginv_v1_gcalz | Calculated gravity effect of sequence down to base Zechstein (mGal). Shown in Figure 23. |
| ginv_v1_gcalc | Calculated gravity field of full model (mGal). Shown in Figure 29 |
| ginv_v1_bzech | Depth to base Zechstein in the gravity model (m) |
| ginv_v1_dep | Depth to the gravity inversion surface (m). Shown in Figure 26. |
| ginv_v1_pzthk | Thickness between base Zechstein and the gravity inversion surface (m). Shown in Figures 27, 31, 33 and 44. |
| ginv_v1_twt | The gravity inversion surface expressed as two-way travel time (ms; positive downward). This grid only covers the offshore area. |
| ginv_v1_twtng | The gravity inversion surface expressed as two-way travel time (ms; negative downward). This grid only covers the offshore area. |
| ginv_v1_bkgrd | Assumed background field (mGal). Shown in Figure 24. |
| ginv_v1_strip | Gravity field after removal of the gravity effect to base Zechstein and the background field (mGal). Shown in Figure 25. |
| ginv_v1_gres | Residual gravity anomalies over the full model (observed gravity - calculated gravity - background) (mGal). Shown in Figure 30. |

Contour shapefiles

| grav_inversion_depth_contours_v1 | Contours of ginv_v1_dep at 500 m intervals. Shown in Figure 26) |
| --- | --- |
| pre_zechstein_thickness_contours_v1 | Contours of ginv_v1_pzthk at 500 m intervals. Shown in Figures 27, 28, 31, 32, 33 and 44. The zero contour has been removed to provide a cleaner display. |